| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | USA. - Please mention Make & Country of Origin against each line item. - Manufacturing date of all items shall not be older than 5 years from the date of delivery. |  |  |  |  |  |
| 10 | 74.10.21.337.1 LINE PIPE 10" SCH-20 SMLS BE LINE PIPE (NPS 10), SCH-20 (6.35 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. LINE PIPE 10" SCH-20 SMLS BE API Grade A Seamless bevelled ends END CAPS MUST BE PROVIDED | 4.00 | PC |  |  |  |
| 20 | 74.10.21.333.1 LINE PIPE DIA 8" SCH 40 X 20' LONG LINE PIPE (NPS 8), SCH-40 (STD) (8.18 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights | 30.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | as per ASME B 36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. |  |  |  |  |  |
| 30 | 74.10.21.299.1 LINE PIPE DIA 1/2" SCH 80 LINE PIPE (NPS 1/2), SCH-80 (XS) (3.73 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. | 60.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 1/2" Schedule: Sch 80 Type: Seamless End Finish: Plain END CAPS MUST BE PROVIDED |  |  |  |  |  |
| 40 | 74.10.21.301.1 LINE PIPE DIA 3/4" SCH 80 LINE PIPE (NPS 3/4), SCH-80 (XS) (3.91 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 3/4" Schedule: Sch 80 | 42.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | Type: Seamless End Finish: Beveled END CAPS MUST BE PROVIDED |  |  |  |  |  |
| 50 | 74.10.21.303.1 LINE PIPE DIA 1" SCH 80 LINE PIPE (NPS 1), SCH-80 (XS) (4.55 mm), seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 1" Schedule: Sch 80 Type: Seamless End Finish: Beveled END CAPS MUST BE PROVIDED | 49.00 | PC |  |  |  |
| 60 | 74.10.21.300.1 LINE PIPE DIA 3/4" SCH-160 20' LONG (P.E | 1.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | LINE PIPE (NPS 3/4), SCH-160 (5.56 mm), seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. LINE PIPE DIA 3/4" SCH-160 20' LONG (P.E) PLAIN END As per API 5L Gr.B or ASTM A-106 Gr.B,Seamless,ASME 36.1 |  |  |  |  |  |
| 70 | 74.10.21.344.1 LINE PIPE 12" SCH 20 20' LONG LINE PIPE (NPS 12), SCH-20 (6.35 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. | 4.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 12" Schedule: Sch 20 Type: Seamless End Finish: Beveled. END CAPS MUST BE PROVIDED |  |  |  |  |  |
| 80 | 74.10.21.309.1 LINE PIPE DIA 2" SCH 80 LINE PIPE (NPS 2), SCH-80 (XS) (5.54 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. | 18.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 2" Schedule: Sch 80 Type: Seamless End Finish: Beveled END CAPS MUST BE PROVIDED |  |  |  |  |  |
| 90 | 74.10.21.313.1 LINE PIPE DIA 3" SCH 40 LINE PIPE (NPS 3), SCH-40 (STD) (5.49 mm), seamless, bevelled ends as per ASME B 16.25, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 | 24.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | years. |  |  |  |  |  |
| 100 | 74.10.21.008.1 LINE PIPE 2" SCH-160, 20'LONG LINE PIPE (NPS 2), SCH-160 , seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 2" Schedule: Sch 160 Type: Seamless End Finish: Plain | 1.00 | PC |  |  |  |
| 110 | 74.10.21.009.1 LINE PIPE 10" SCH-XXS, 20'LONG LINE PIPE (NPS 10), SCH-XXS , seamless, plain ends, length 20 | 1.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 10" Schedule: XXS Type: Seamless End Finish: Plain |  |  |  |  |  |
| 120 | 74.10.21.010.1 LINE PIPE 2" SCH-XXS, 20'LONG LINE PIPE (NPS 2), SCH-XXS , seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to | 1.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 2" Schedule: XXS Type: Seamless End Finish: Plain |  |  |  |  |  |
| 130 | 74.10.21.011.1 LINE PIPE 3" SCH-XXS, 20'LONG LINE PIPE (NPS 3), SCH-XXS , seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B36.10, origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided | 1.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 3" Schedule: XXS Type: Seamless End Finish: Plain |  |  |  |  |  |
| 140 | 74.10.21.012.1 LINE PIPE 4" SCH-XXS, 20'LONG LINE PIPE (NPS 4), SCH-XXS , seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B | 1.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dia(NPS): 4" Schedule: XXS Type: Seamless End Finish: Plain |  |  |  |  |  |
| 150 | 74.10.21.013.1 LINE PIPE 1" SCH-XXS, 20'LONG LINE PIPE (NPS 1), SCH-XXS , seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 1" Schedule: XXS Type: Seamless End Finish: Plain | 1.00 | PC |  |  |  |
| 160 | 74.10.21.307.1 LINE PIPE DIA 1-1/2" SCH 80 | 6.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | NEW SPECS PROVIDED BY MR IMRAN AHMED THROUGH EMAIL DATED 24.02.2023 LINE PIPE (NPS 1 1/2), SCH-80 (XS) (5.08 mm), seamless, plain ends, length 20 ft, fully killed carbon steel, API 5L or SA-106 Gr.B, compliance with API 5L or ASME sec-II part A, dimensions & weights as per ASME B 36.10, Origin of material must be W.europe, UK, USA, Japan, south korea. OEM as per PRL attached approved vendor list. Pipes must be provided with end caps at both ends to avoid damage of ends during transportation & handling. Material shall be as per LATEST standard/code. In case the latest standard/code is less than two years old, the material shall comply to previous edition. Older material (compliant to older editions of codes/standards) shall not be acceptable in any case. pipe material shall comply to NACE MR-0175 and MR-0103. MTCs EN 10204: 3.1 of finished product shall be provided for material with proper traceability of material against MTCs. Material should be free from burrs, damages, dent, cuts, gouges & corrosion etc. Manufacturing date of pipes shall not be older than 05 years. Line Pipe. Specification: API 5L OR EQUIVALENT Grade: B Dia(NPS): 1-1/2" Schedule: Sch 80 Type: Seamless End Finish: Beveled END CAPS MUST BE PROVIDED |  |  |  |  |  |
| 170 | 74.10.21.014.1 LINE PIPE 1-1/2" SCH-80, 20'LONG (108-FN Material = SA-333 Gr. 6 Length = 20ft. | 10.00 | PC |  |  |  |
| 180 | 74.10.21.015.1 | 10.00 | PC |  |  |  |

| Sr. No. | Material / Description | Qty. | Unit | Unit Rate | S. Tax | Total Price |
| --- | --- | --- | --- | --- | --- | --- |
|  | SS LINE PIPE 2" SCH-40, 6M LONG Material = AISI304 Length = 6m |  |  |  |  |  |
| 190 | 74.10.21.016.1 SS LINE PIPE 1" SCH-40, 6M LONG Material = AISI304 Length = 6m | 10.00 | PC |  |  |  |
| 200 | 74.10.21.017.1 SS LINE PIPE 3/4" SCH-40, 6M LONG Material = AISI304 Length = 6m | 1.00 | PC |  |  |  |
|  | Grand Total : |  |  |  |  |  |

Origin shall be Western Europe, Japan, South Korea, U.K.

,

& USA

NIPPON STEEL & SUMITOMO		 	JAPAN

2 TENARIES 						ITALY 			www.tenaris.com

3 V & M 						FRANCE

4 DALMIRE 						ITALY

5 ARCELOR MITTAL LUXEMBORG

6 TUBOS 						SPAIN

7 BENTELER 						GERMANY

8 VALLOUREC					FRANCE

9 Interpipe						 Ukraine